MAE 158 Extra Credit Assignment

Thomas Slagle (72602272)

Part 1

Problem Statement

The Python script written for part one of the extra credit assignment evaluates performance characteristics of the DC – 9 – 30 commercial airliners over a range of flight speed values at a 31,000 ft cruising altitude. This aircraft was evaluated for speeds between 250 and 575 knots true airspeed. The aircraft parameters used in the script are shown in Table 1. The values obtained from the Python script are shown in Table 2 and plotted in Figures 1, 2, and 3. Lastly, the complete Python script is shown in Appendix A.

Table 1 presents the relevant parameters that were input in as variables in the Python script. The values listed in Table 1 were used to derive all other necessary inputs for the script such as the dynamic viscosity of air, the speed of sound, and the Reynolds number.

Table 1: DC – 9 – 30 Parameters

| Parameter | Value |
| --- | --- |
| General Aircraft Properties |  |
| Cruise Weight (Wc) | 97,000 [lbs] |
| Descent Weight (Wd) | 82,000 [lbs] |
| Engine Properties |  |
| Specific Fuel Consumption (Ct) | 0.82 [lblb-h] |
| Sea-Level Thrust (Tsl) | 14,500 [lbs] |
| Environment Properties |  |
| Altitude (h) | 31,000 [ft] |
| Temperature (T) | 399.67 [°R] |
| Gas Constant (R) | 1718 [ft*lbfslug*°R] |
| Heat Capacity Ratio (γ) | 1.4 |
| Sea Level Density (ρsl) | 0.002377 [slugsft3] |
| Altitude Density (ρ) | 0.000876 [slugsft3] |
| Wing Properties |  |
| Planform Area (Sw) | 1,000 [ft2] |
| Span (bw) | 93.2 [ft] |
| Root Chord (crw) | 17.8 [ft] |
| Sweep Angle (Λw) | 24.5 [deg] |
| Average Thickness Ratio (tcw) | 0.106 |
| Taper Ratio (σw) | 0.2 |
| Wing Area Covered by Fuselage (Scovw) | 17% |
| Vertical Tail Properties |  |
| Planform Area (Sv) | 161 [ft2] |
| Root Chord (crv) | 15.5 [ft] |
| Sweep Angle (Λv) | 43.5 [deg] |
| Average Thickness Ratio (tcv) | 0.09 |
| Taper Ratio (σv) | 0.80 |
| Horizontal Tail Properties |  |
| Planform Area (Sh) | 261 [ft2] |
| Root Chord (crh) | 11.1 [ft] |
| Sweep Angle (Λh) | 31.6 [deg] |
| Average Thickness Ratio (tch) | 0.09 |
| Taper Ratio (σh) | 0.35 |
| Fuselage Properties |  |
| Wetted Area (Swetf) | 3,280 [ft2] |
| Diameter (Ø) | 11.5 [ft] |
| Length (lf) | 107 [ft] |
| Nacelles Properties |  |
| Wetted Area (Swetn) | 455 [ft2] |
| Finesse Ratio (fin) | 5.0 |
| Length (ln) | 16.8 [ft] |
| Pylons Properties |  |
| Wetted Area (Swetp) | 117 [ft2] |
| Chord (Cp) | 16.2 [ft] |
| Sweep Angle (Λp) | 0 [deg] |
| Average Thickness Ratio (tcp) | 0.06 |
| Taper Ratio (σp) | 1.0 |
| Flap Hinge Fairings |  |
| Flat Plate Drag Area (fFH) | 0.15 [ft2] |

Table 2 and figures 1, 2, and 3 present the values and plots generated from the code. The values were all generated using the methods learned in class.

Table 2: Results obtained from the Python Script

| Parameter | Calculated Value |
| --- | --- |
| Total Parasitic Drag Coefficient | 0.0217 – 0.0197 across airspeed range |
| Total Flat Plate Drag Area | 21.75 – 19.69 [ft²] across airspeed range |
| Maximum Range | 1988.46 [miles] @ 509.28 [kts] |
| Maximum Endurance | 1.21 [hrs] @ 383.06 [kts] |

Figure 1. Plot showing total, induced, and parasitic drags versus airspeed.

Figure 2. Plot showing power required and power available from the jet engines versus airspeed.

Figure 3. Plot showing rate of climb versus airspeed.

Part 2

Problem Statement

The Python script written for part one of the extra credit assignment evaluates performance characteristics of the Pipistrel Alpha Electro aircraft over a range of flight speed values at a 9,000 ft cruising altitude. This aircraft was evaluated for speeds between 25 and 200 knots true airspeed. The aircraft parameters used in the script are shown in Table 3. The values obtained from the Python script are shown in Table 4 and plotted in Figures 5, 6, and 7. Lastly, the complete Python script is shown in Appendix B.

Table 3 presents the relevant parameters that were input in as variables in the Python script. Values that were not available from Pipistrel directly were evaluated using the ImageJ program, which relates a known pixel length in an image to a known distance, the results are shown in Figure 4. The thickness ratio was assumed to be constant across the wing and tails, respectively. Taper ratio was difficult to estimate from the images available online and was thus assumed to be 1 for all geometries that needed it. The values listed in Table 3 were used to derive all other necessary inputs for the script such as the dynamic viscosity of air, the speed of sound, and the Reynolds number.

Figure 4. ImageJ results for aircraft parameters.

Table 3: Pipistrel Alpha Electro Parameters

| Parameter | Value |
| --- | --- |
| General Aircraft Properties |  |
| Gross Weight (Wg) | 1,214 [lbs] |
| Battery Weight (Wb) | 277 [lbs] |
| Engine Properties |  |
| Cell Energy Density (Cb) | 130 [Whlb] |
| Propellor-Motor Efficiency (η) | 86% |
| Battery Efficiency (bat) | 180 [Wlb-T] |
| Motor Shaft Power (Pshaft) | 50,000 [W] |
| Environment Properties |  |
| Altitude (h) | 9000 [ft] |
| Temperature (T) | 486.61 [°R] |
| Gas Constant (R) | 1718 [ft*lbfslug*°R] |
| Heat Capacity Ratio (γ) | 1.4 |
| Sea Level Density (ρsl) | 0.002377 [slugsft3] |
| Altitude Pressure (P) | 1512.9 [lbft^2] |
| Wing Properties |  |
| Planform Area (Sw) | 102.4 [ft2] |
| Span (bw) | 34.5 [ft] |
| Root Chord (crw) | 2.896 [ft] |
| Sweep Angle (Λw) | 0 [deg] |
| Thickness Ratio (tcw) | 0.1775 |
| Taper Ratio (σw) | 1 |
| Vertical Tail Properties |  |
| Planform Area (Sv) | 11.8 [ft2] |
| Root Chord (crv) | 4.109 [ft] |
| Sweep Angle (Λv) | 0 [deg] |
| Thickness Ratio (tcv) | 0.10 |
| Taper Ratio (σv) | 1 |
| Horizontal Tail Properties |  |
| Planform Area (Sh) | 11.6 [ft2] |
| Root Chord (crh) | 2.240 [ft] |
| Sweep Angle (Λh) | 0 [deg] |
| Thickness Ratio (tch) | 0.10625 |
| Taper Ratio (σh) | 1 |
| Fuselage Properties |  |
| Length (lf) | 21.4 [ft] |
| Diameter (Ø) | 3.5 [ft] |
| Landing Gear |  |
| Flat Plate Drag Area (fLG) | 0.15 [ft2] |

Table 4 and figures 5, 6, and 7 present the values and plots generated from the code. The values were all generated using the methods learned in class. The results obtained from the code were compared to the spec sheet for this aircraft from Pipistrel’s website. The results matched well for the flight speed range, the listed maximum fight speed was 135 kts with a nominal cruising speed of 85 kts; however, the max range and endurance values generated from the Python script were much higher than their listed values. This is likely due to the cell efficiency given in the assignment description being higher than what pipistrel offers.

Table 4: Results obtained from the Python Script

| Parameter | Calculated Value |
| --- | --- |
| Total Parasitic Drag Coefficient | 0.0338 – 0.0224 across airspeed range |
| Total Flat Plate Drag Area | 3.46 – 2.29 [ft²] across airspeed range |
| Maximum Range | 529.18 [miles] @ 75.98 [kts] |
| Maximum Endurance | 4.76 [hrs] @ 56.36 [kts] |

Figure 5. Plot showing total, induced, and parasitic drags versus airspeed.

Figure 6. Plot showing power required and power available from the propellor motor versus airspeed.

Figure 7. Plot showing rate of climb versus airspeed; the graph becomes negative one the power required is greater than the power available (shown in Figure 6).

Appendix A

import numpy as np

from matplotlib import pyplot as plt

#Functions

def Re(R,L):

return (R*L)

def C_th(C_r, sigma):

return C_r * sigma

def MAC_exp(Y,C_r,sigma,B):

C_t = C_th(C_r, sigma)

C_r_fuse = C_r - (C_r - C_t)*Y/(B/2)

sigma_exp = C_t/C_r_fuse

return (2/3*C_r_fuse*(1+sigma_exp-sigma_exp/(1+sigma_exp)))

def MAC(C_r,sigma):

return (2/3*C_r*(1+sigma-sigma/(1+sigma)))

#Parasitic Drag Equation (Calc from Nabil Discussion)

def c_f(R):

return 0.208*R**-0.27845 + 0.00101

#0.1944*R**-0.2734 + 0.0009782

#e_lambda_zero Equation (calc from Nabil Discussion)

def e(Ar, C_d_p, _lambda):

e_straight = np.abs(1.78*(1 - 0.045*Ar**0.68) - 0.64)

e_30 = np.abs(4.61*(1 - 0.045*Ar**0.68)*(np.cos(np.radians(_lambda)))**0.15 - 3.1)

e_mid = np.abs((e_30 - e_straight)/(np.radians(_lambda) - 0))

if (_lambda == 0):

return e_straight

elif (_lambda >= 30):

return e_30

else:

return e_mid

#0.9423 + 0.00041*Ar + 4.591*C_d_p - 6.878e-06*Ar**2 - 1.348*Ar*C_d_p - 242.9 *C_d_p**2 + 0.01989*Ar**2*C_d_p + 11.86*Ar*C_d_p**2 + 3483*C_d_p**3

def kappa(M, Lambda, TC):

z = ((2-M**2)*np.cos(np.radians(Lambda)))/(np.sqrt(1 - M**2*np.cos(np.radians(Lambda))))

return 1 + z*TC + 100*TC**4

#Given Parameters

#General Plane Properties

W_i = 97000 #lbs

W_d = 82000 #lbs

C_t = 0.82 #lb/lb-h

W_ker = 6.7 #lb/gal

#Wing Geometry

Lambda_w = 24.5 #deg

tc_w = 0.106

b_w = 93.2 #ft

S_w = 1000 #ft2

sigma_w = 0.2

c_r_w = 17.8 #ft

S_covered_w = 0.17

#Vertical Tail

Lambda_v = 43.5 #deg

tc_v = 0.09

S_v = 161 #ft2

sigma_v = 0.80

c_r_v = 15.5

#Horizontal Tail

Lambda_h = 31.6 #deg

tc_h = 0.09

S_h = 261 #ft2

sigma_h = 0.35

c_r_h = 11.1 #ft

#Fuselage Geometry

l = 107 #ft

dia = 11.5 #ft

S_wet = 3280 #ft2

#Pylons Geometry

S_wet_p = 117 #ft2

tc_p = 0.06

Lambda_p = 0 #deg

sigma_p = 1.0

c_p = 16.2 #ft

#Nacelles Geometry

S_wet_n = 455 #ft2

fin_ratio = 5.0

L_n = 16.8 #ft

#Flap hinge Fairing

delta_f_FH = 0.15 #ft2

#Environmental Variables

h = 31000 #ft

#M = 0.78

T_f = -60 #F

T = -60 + 459.67

rho_sl = 0.002377

rho = 0.000876 #slugs/ft3

mu = 0.3170*(T**(3/2))*(734.7/(T+216))*(1/10**10) #3.04e-07

#print(mu)

gamma = 1.4

R = 1718

a = np.sqrt(gamma*R*(T))

#print(a)

#V_0 = M*a

T_sl = 14500

T = T_sl*(rho/rho_sl)

print(T)

#Arrays

V_array_kts = np.linspace(250,575,1000)

D_i_array = np.array([])

D_p_array = np.array([])

D_tot_array = np.array([])

ld_array = np.array([])

R_array = np.array([])

E_array = np.array([])

P_req_array = np.array([])

P_ava_array = np.array([])

P_exc_array = np.array([])

roc_array = np.array([])

i = 0

for i in range (len(V_array_kts)):

V = V_array_kts[i]*1.68781

M = V/a

q = (rho*V**2)/2

#q = gamma/2*601.61*M**2

Re_l = rho*V/mu #Rn/ft

#Wing

mac_w = MAC_exp(dia, c_r_w, sigma_w, b_w)

re_w = Re(Re_l, mac_w)

cf_w = c_f(re_w)

kappa_w = kappa(M, Lambda_w, tc_w)

delta_f_w = (1.02*2*S_w*(1-S_covered_w)*cf_w*kappa_w)

#Horizontal Tail

mac_h = MAC(c_r_h, sigma_h)

re_h = Re(Re_l, mac_h)

cf_h = c_f(re_h)

kappa_h = kappa(M, Lambda_h, tc_h)

delta_f_h = (1.02*2*S_h*cf_w*kappa_h)

#Vertical Tail

mac_v = MAC(c_r_v, sigma_v)

re_v = Re(Re_l, mac_v)

cf_v = c_f(re_v)

kappa_v = kappa(M, Lambda_v, tc_v)

delta_f_v = (1.02*2*S_v*cf_v*kappa_v)

#Pylons

re_p = Re(Re_l, c_p)

cf_p = c_f(re_p)

kappa_p = kappa(M, Lambda_p, tc_p)

delta_f_p = (S_wet_p*cf_p*kappa_p)

#Fuselage

re_f = Re(Re_l, l)

fin_F = l/dia

kappa_f = 1.11

cf_f = c_f(re_f)

delta_f_f = S_wet*cf_f*kappa_f

#Nacelles

re_n = Re(Re_l, L_n)

cf_n = c_f(re_n)

kappa_n = 1.29

delta_f_n = S_wet_n*cf_n*kappa_n

#Total Parasite drag

f = 1.10*(delta_f_w + delta_f_f + delta_f_h + delta_f_v + delta_f_n + delta_f_p + delta_f_FH)

C_d_p = f/S_w

AsR = b_w**2/S_w

C_l = W_i/(q*S_w)

#print(C_l)

C_d_i = C_l**2/(np.pi*AsR*e(AsR, C_d_p, Lambda_w))

D_i = (W_i/b_w)**2/(np.pi*q*e(AsR, C_d_p, Lambda_w))

D_i_array = np.append(D_i_array,[D_i])

D_p = f*q

D_p_array = np.append(D_p_array,[D_p])

D_tot = D_p + D_i

C_d = D_tot/(q*S_w)

D_tot_array = np.append(D_tot_array,[D_tot])

ld = D_tot/W_i

ld_array = np.append(ld_array, [ld])

P_req = (1/550)*np.sqrt(2*W_i**3/(rho_sl*S_w))*1/(C_l**(3/2)/C_d) #D_tot*V/550#

P_req_array = np.append(P_req_array, [P_req])

R_jet = 2/C_t * np.sqrt(2/(rho*S_w))*np.sqrt(C_l)/(C_d_p+C_d_i)*(np.sqrt(W_i)-np.sqrt(W_d))

R_array = np.append(R_array, [R_jet])

E_jet = 1/C_t*(C_l/C_d)*np.log10(W_i/W_d)

E_array = np.append(E_array, [E_jet])

P_ava = T*V/550

P_ava_array = np.append(P_ava_array, [P_ava])

P_exc_array = np.append(P_exc_array, [(P_ava-P_req)*4])

roc = (T_sl/W_i - 1/(C_l/C_d))*V*60

roc_array = np.append(roc_array,[roc])

print('Your max range is: \n' +  str(np.max(R_array)) + ' miles\nThis value occurs at:\n' + str(V_array_kts[np.argmax(R_array)]) + ' kts\n\n')

print('Your max endurance is:\n' + str(np.max(E_array)) + ' hr\n' + str(V_array_kts[np.argmax(E_array)]) + ' kts')

plt.plot(V_array_kts, D_tot_array, label = 'Total Drag')

plt.plot(V_array_kts, D_i_array, label = 'Induced Drag')

plt.plot(V_array_kts, D_p_array, label = 'Parasitic Drag')

plt.xlabel('Airspeed (Kts)')

plt.ylabel('Drag (lbs)')

plt.title('Drag v Airspeed')

plt.legend()

plt.show()

plt.close()

plt.plot(V_array_kts, P_req_array, label = 'Power Required')

plt.plot(V_array_kts, P_ava_array, label = 'Power Available')

#plt.plot(V_array_kts, P_exc_array, label = 'Excess Power')

#ax2 = plt.secondary_yaxis("right", functions=(V_array_kts, P_exc_array))

plt.xlabel('Airspeed (Kts)')

plt.ylabel('Power (hp)')

plt.title('Power Required v Airspeed')

plt.legend()

plt.show()

plt.close()

plt.plot(V_array_kts, roc_array)

plt.xlabel('Airspeed (Kts)')

plt.ylabel('Rate of Climb (ft/min)')

plt.title('Rate of Climb v Airspeed')

plt.show()

Appendix B

###### PART 2 #####

import numpy as np

from matplotlib import pyplot as plt

#Functions

def Re(R,L):

return (R*L)

def C_th(C_r, sigma):

return C_r * sigma

def MAC_exp(Y,C_r,sigma,B):

C_t = C_th(C_r, sigma)

C_r_fuse = C_r - (C_r - C_t)*Y/(B/2)

sigma_exp = C_t/C_r_fuse

return (2/3*C_r_fuse*(1+sigma_exp-sigma_exp/(1+sigma_exp)))

def MAC(C_r,sigma):

return (2/3*C_r*(1+sigma-sigma/(1+sigma)))

#Parasitic Drag Equation (Calc from Nabil Discussion)

def c_f(R):

return 0.208*R**-0.27845 + 0.00101

#0.1944*R**-0.2734 + 0.0009782

#e_lambda_zero Equation (calc from Nabil Discussion)

def e(Ar, C_d_p, _lambda):

e_straight = np.abs(1.78*(1 - 0.045*Ar**0.68) - 0.64)

return e_straight

def kappa(M, Lambda, TC):

z = ((2-M**2)*np.cos(np.radians(Lambda)))/(np.sqrt(1 - M**2*np.cos(np.radians(Lambda))))

return 1 + z*TC + 100*TC**4

#Given Parameters

C_bat = 130 #Wh/lb

bat = 180 #W/lb-T

eta_p = 0.86 #https://en.wikipedia.org/wiki/Propulsive_efficiency

#Basing my design on the Alpha-Electro Plane

#Many basic dimensions: https://www.pipistrel-aircraft.com/aircraft/electric-flight/alpha-electro/

#Measurements taken using imageJ

W_bat = 277 # lbs

Wh_bat = W_bat * C_bat #Wh

W_gross = 1214 #lb

P_shaft = 50000 #Watts

#Wing Geometry

Lambda_w = 0 #deg

tc_w = 0.1775

b_w = 34.5 #ft

S_w = 102.4 #ft2

sigma_w = 1

c_r_w = 2.896 #ft

#Vertical Tail

Lambda_v = 0 #deg

tc_v = 0.10

S_v = 11.8 #ft2

sigma_v = 0.654

c_r_v = 4.109

#Horizontal Tail

Lambda_h = 0 #deg

tc_h = 0.10625

S_h = 11.6 #ft2

sigma_h = 1

c_r_h = 2.240 #ft

#Fuselage Geometry

l = 21.4 #ft

dia = 3.5 #ft

S_wet = np.pi * dia * (l - 1.3*dia) #ft2 #calculated from https://onlinelibrary.wiley.com/doi/pdf/10.1002/9781118568101.app1

#Landing Gear

delta_f_LG = 0.15 #ft2

#Environmental Variables

gamma = 1.4

R = 1718

h = 9000 #ft

T = 486.61 #R

rho_sl = 0.002377

P = 1512.9

rho = P/(R*T) #slugs/ft3

mu = 0.3170*(T**(3/2))*(734.7/(T+216))*(1/10**10) #3.04e-07

a = np.sqrt(gamma*R*(T))

#print(T)

#Arrays

V_array_kts = np.linspace(25,200,1000)

D_i_array = np.array([])

D_p_array = np.array([])

D_tot_array = np.array([])

ld_array = np.array([])

R_array = np.array([])

E_array = np.array([])

P_req_array = np.array([])

P_ava_array = np.array([])

P_exc_array = np.array([])

roc_array = np.array([])

i = 0

for i in range (len(V_array_kts)):

V = V_array_kts[i]*1.68781

M = V/a

q = (rho*V**2)/2

#q = gamma/2*601.61*M**2

Re_l = rho*V/mu #Rn/ft

#Wing

mac_w = MAC_exp(dia, c_r_w, sigma_w, b_w)

re_w = Re(Re_l, mac_w)

cf_w = c_f(re_w)

kappa_w = kappa(M, Lambda_w, tc_w)

delta_f_w = (1.02*2*S_w*cf_w*kappa_w)

#print(delta_f_w)

#Horizontal Tail

mac_h = MAC(c_r_h, sigma_h)

re_h = Re(Re_l, mac_h)

cf_h = c_f(re_h)

kappa_h = kappa(M, Lambda_h, tc_h)

delta_f_h = (1.02*2*S_h*cf_w*kappa_h)

#Vertical Tail

mac_v = MAC(c_r_v, sigma_v)

re_v = Re(Re_l, mac_v)

cf_v = c_f(re_v)

kappa_v = kappa(M, Lambda_v, tc_v)

delta_f_v = (1.02*2*S_v*cf_v*kappa_v)

#Fuselage

re_f = Re(Re_l, l)

fin_F = l/dia

kappa_f = 1.11

cf_f = c_f(re_f)

delta_f_f = S_wet*cf_f*kappa_f

#Total Parasite drag

f = 1.10*(delta_f_w + delta_f_f + delta_f_h + delta_f_v + delta_f_LG)

C_d_p = f/S_w

AsR = b_w**2/S_w

C_l = W_gross/(q*S_w)

#print(C_l)

C_d_i = C_l**2/(np.pi*AsR*e(AsR, C_d_p, Lambda_w))

D_i = (W_gross/b_w)**2/(np.pi*q*e(AsR, C_d_p, Lambda_w))

D_i_array = np.append(D_i_array,[D_i])

D_p = f*q

D_p_array = np.append(D_p_array,[D_p])

D_tot = D_p + D_i

C_d = D_tot/(q*S_w)

D_tot_array = np.append(D_tot_array,[D_tot])

ld = D_tot/W_gross

ld_array = np.append(ld_array, [ld])

P_req = np.sqrt(2*W_gross**3/(rho_sl*S_w))*1/(C_l**(3/2)/C_d) #(1/550)*

P_req_array = np.append(P_req_array, [P_req])

E_prop = Wh_bat/P_req

E_array = np.append(E_array, [E_prop])

R_prop = E_prop * V

R_array = np.append(R_array, [R_prop])

P_ava = P_shaft * eta_p

P_ava_array = np.append(P_ava_array, [P_ava])

P_exc_array = np.append(P_exc_array, [(P_ava-P_req)*4])

roc = (P_ava-P_req)/W_gross*60

roc_array = np.append(roc_array,[roc])

print('Your max range is: \n' +  str(np.max(R_array)) + ' miles\nThis value occurs at:\n' + str(V_array_kts[np.argmax(R_array)]) + ' kts\n\n')

print('Your max endurance is:\n' + str(np.max(E_array)) + ' hr\n' + str(V_array_kts[np.argmax(E_array)]) + ' kts')

#PLOTS

plt.plot(V_array_kts, D_tot_array, label = 'Total Drag')

plt.plot(V_array_kts, D_i_array, label = 'Induced Drag')

plt.plot(V_array_kts, D_p_array, label = 'parasitic Drag')

plt.xlabel('Airspeed (Kts)')

plt.ylabel('Drag (lbs)')

plt.title('Drag v Airspeed')

plt.legend()

plt.show()

plt.close()

plt.plot(V_array_kts, P_req_array, label = 'Power Required')

plt.plot(V_array_kts, P_ava_array, label = 'Power Available')

#plt.plot(V_array_kts, P_exc_array, label = 'Excess Power')

#ax2 = plt.secondary_yaxis("right", functions=(V_array_kts, P_exc_array))

plt.xlabel('Airspeed (Kts)')

plt.ylabel('Power (hp)')

plt.title('Power Required v Airspeed')

plt.legend()

plt.show()

plt.close()

plt.plot(V_array_kts, roc_array)

plt.xlabel('Airspeed (Kts)')

plt.ylabel('Rate of Climb (ft/min)')

plt.title('Rate of Climb v Airspeed')

plt.show()